Principles of Pharmacy Practice	Chapter 3: Pharmaceutical Measurement 2024 – 2025

Dr. Zainab Thabit Saleh

# Objectives:

Upon successful completion of this chapter, the student will be able to:

- Describe instruments for volumetric measurement and characterize their differences in application and accuracy.

- Describe the correct procedure when using a pharmaceutical balance.

- Define sensitivity requirement and apply it in calculations.

- Perform calculations by the aliquot method.

- Demonstrate an understanding of percentage of error in pharmaceutical measurement.

# Why we need to learn about pharmaceutical measurements?

The role of the pharmacist in providing pharmaceutical care includes the ability and responsibility to compound—that is, to accurately weigh, measure volume, and combine individual therapeutic and pharmaceutical components in the formulation and preparation of prescriptions and medication orders

# Measurement of volume

The selection of measuring instrument should be based on the level of precision required

Select the graduate with a capacity equal to or just exceeding the volume to be measured

- The narrower the bore or chamber, the lesser the error in reading the meniscus and the more accurate the measurement

# Measurements of weight

Sensitivity requirement: The load that will cause a change of one division on the index plate of the balance.

- Class A prescription balance has a sensitivity requirement (SR) of 6 milligrams or less with no load and with a load of 10 grams in each pan.

- To avoid errors of greater than 5% when using this balance, the pharmacist should not weigh less than 120 milligrams of material (i.e., a 5% error in a weighing of 120 milligrams is 6 milligrams).

sensitivity requirement may be determined by the following procedure:

- Level the balance.

- Determine the rest point of the balance.

- Determine the smallest weight that causes the rest point to shift one division on the index plate.

- Most commercially available Class A balances have a maximum capacity of 120 grams

# Electronic analytical balance

- Capable of weighing accurately 0.1 milligram

- Self-calibrating

- Equipped with convenient digital readout features.

- The usual maximum capacities for balances of this precision range from about 60 grams to 210 grams depending upon the model

# Aliquot method of weight and measurement

Why use this method?

When a degree of precision in measurement required is beyond the capacity of the available instrument.

Aliquot is a fraction, portion, or part that is contained an exact number of times in another.

Step 1: Calculate the smallest quantity of a substance that can be weighed on a balance with the desired precision

Step 2: Weigh an amount of the drug equal to or greater than the least weighable amount and calculate the enlargement factor: weighed amount/desired amount

Step 3: choose aliquot weight equal to or larger than the smallest weighable amount

Step 4: determine mixture weight = aliquot X factor

Step 5: calculate weight of diluent needed = mixture – drug

100% ×6 = 125 mg, error % = 4.8%

ⅇrror %

↑amount to be weighed → ↓error

Step 2: weigh 125 mg, factor = 125/5 = 25

Step 3: choose aliquot weight equal to or larger than the smallest weighable amount, choose 120 mg

Step 4: determine mixture weight = aliquot X factor = 120 X 25 = 3000

Step 5: calculate weight of diluent needed = mixture – drug = 3000 – 125 = 2875 mg

Sensitivity requirement = 6.5 mg

𝑠𝑚𝑎𝑙𝑙𝑒𝑠𝑡 𝑞𝑢𝑎𝑛𝑡𝑖𝑡𝑦 = 100% ×𝑆𝑅  = 130 mg

ⅇrror %

- Smallest amount = 130 mg

- Weight 150 mg atropine sulfate (130/15 = 8.6 so we choose 150 to get an easy-to-use factor) factor =150/15 = 10

- Choose aliquot weight 130 mg

- Mixture weight = aliquot X factor = 130 X 10 = 1300

- Calculate weight of diluent needed = mixture – drug = 1300 mg – 150 = 1150 mg

- Check to make sure 130 mg contains 15 mg drug: 150 mg drug	1300 mg mixture

?	130 mg mixture	? = 150 X 130 /1300 = 15 mg Important note: You can choose to weigh

SR = 6.5

Error 2%

Amount drug needed = 20 mg

- Least weighable amount = 100 X 6.5 / 2 = 385 mg

- Weight 400 mg atropine sulfate (400/20 = 20)

- Choose aliquot weight 385 mg (you can choose 400 mg)

- Mixture weight = aliquot X factor = 385 X 20 = 7700

- Calculate weight of diluent needed = mixture – drug = 7700 mg – 400 = 7300 mg

- Check to make sure 385 mg contains 20 mg drug:

400 mg	3700 mg

20 mg	?	, aliquot = 7300 X 20 /400 = 385 mg of the mixture to contain

20 mg substance

Or: if you choose to use 400 mg as the aliquot weight

(20 X 400) – 400 = 8000 – 400 = 7600 mg diluent,

400g	8000 mg

20 mg	? aliquot = 8000 X 20 /400 = 400 mg

# Measuring volume by the aliquot method

Step 1. Select a multiple of the desired quantity that can be measured with the required precision.

Step 2. Dilute the multiple quantity with a compatible diluent (usually a solvent for the liquid to be measured) to an amount evenly divisible by the multiple selected.

Step 3. Measure the aliquot of the dilution that contains the quantity originally desired

Amount drug to be weighed > 80 mg, weigh 90 mg Dissolve in 12 ml

12 ml	90 mg

?	15 mg	? = 12 ml X 15 mg / 90 mg = 2 ml

- Weigh 90 mg drug

- Dissolve on 12 ml cherry syrup

- Use 2 ml to fill the prescription

# Percentage of error

measurements in the community pharmacy are never absolutely accurate

- Need to recognize the limitations of the instruments used and the magnitude of the errors that may be incurred.

- Need to know

- the apparent weight or volume measured

- the possible excess or deficiency in the actual quantity obtained.

Percentage of error the maximum potential error multiplied by 100 and divided by the quantity desired.

𝐸𝑟𝑟𝑜𝑟 × 100%

𝑃𝑒𝑟𝑐𝑒𝑛𝑡𝑎𝑔𝑒 𝑜𝑓 𝑒𝑟𝑟𝑜𝑟 = 𝑄𝑢𝑎𝑛𝑡𝑖𝑡𝑦 𝑑𝑒𝑠𝑖𝑟𝑒𝑑

## Using a graduated cylinder, a pharmacist measured 30 milliliters of a liquid. On subsequent examination, using a narrow-gauge burette, it was determined that the pharmacist had actually measured 32 milliliters. What was the percentage of error in the original measurement?

Answer:

32 ml - 30 ml = 2 ml, the volume of error (2 mL X 100%)/30 mL =  6.7%

## A prescription calls for 800 milligrams of a substance. After weighing this amount on a balance, the pharmacist decides to check by weighing it again on a more sensitive balance, which registers only 750 milligrams. Because the first weighing was 50 milligrams short of the desired amount, what was

the percentage of error?

Answer:

50 mg the weight of error

(50 mg X 100%)/800 mg = 6.25%

The smallest quantity that should be weighed on the balance:

Quantity desired: 5 mg

100% × 4 𝑚𝑔

5%	= 80 𝑚𝑔

Multiple factor selected: 20, weigh 100 mg of the drug (20X5 = 100)

Aliquot portion selected: 90 mg

Hydromorphone hydrochloride (20 X 5) =	100 mg

Lactose		= 1700 mg Aliquot mixture	= 1800 mg

Proof: 1/20 of the drug	100 mg/20 = 5 mg 1/20 of the diluent 1700 mg /20 = 85 mg

90 mg aliquot

Multiple factor selected: 5 Aliquot volume selected: 2 ml

Measure 3 ml of the coloring solution (0.6 ml X 5 = 3 ml) Dilute to 10 ml

Measure 2 ml Proof:

10 ml dilution	3 ml coloring agent

?	0.6 ml coloring agent

? = 10 ml X 0.6 ml/ 3 ml = 2 ml

# HW: A pharmacist needed to weigh 20 mg of a drug using a balance with a sensitivity requirement of 6.5 mg and error not greater than 2%. This was his plan: weigh 340 mg drug, dilute with 5440 mg diluent to make 5780 mg, weigh 325 mg. Is his work correct?

Error = quantity measured – desired quantity

= 28.35 g – 31.1 g = - 2.75 g = 2.75 g (negative sign means we are measuring less than needed)

Percentage of error = (error X 100%) / desired quantity

= (2.75 g X 100% ) / 31.1 g = 8.84 %